## Lesson 25 – Keyring Activity Sheet

This tutorial is based upon those created by Warwick University for their STEM outreach programme and simplified. The original tutorial along with many other excellent tutorials are available at

https://warwick.ac.uk/fac/sci/wmg/about/outreach/3d_design_printing/cad_tutorials/

Step 1 Set your view to TOP

Step 2 Draw a simple rectangle of 65 x 25mm

Step 3 from the sketch menu chose fillets to curve the edges

Select the corner you wish to curve

And make the radius 6mm – do this for all 4 corners

Step 4 use CTRL C and CTRL V to make a copy of you shape

And move it below the first

Step 5 From the primitives option draw a 4mm circle where you want your keyring to mount

Step 6 – From the construction menu select extrude

Select the shape and extrude it 2mm up

Step 7 From the sketch menu select offset

And choose the outside edge of the bottom shape

Set the offset to 1.5mm

Step 8 – Select the offset you just created and extrude it up by 1.5mm

Step 9 choose the snap tool

Select your rim and then the top face to join them together

Step 10 Choose the text option

Specify the starting point for your text

Type, size and position before pressing OK

Step 11 Select the text, click the cog and choose extrude text

Extrude up to 1.5mm

Save your file to you PC and Export it as an STL file for printing

Remember to combine the objects